# Robot Operation Document

This document is based on basic usage of teach pendant for controlling robot movements. It particularly covers the following topics:

- Starting of Robot

- Robot Homing Procedure

- Running teach pendant program

- Operator Box usage

# Starting of Robot

- Switch on the main power supply.

- Switch on the power supply to the robot controller by turning the red knob in clockwise direction.

- Press “Controller on” (green color button on the controller). Note: In case, green light doesn’t get on then check emergency stop buttons (red color) on the controller, operator box and the teach pendant. They should be released (move in anti-clockwise direction) to switch on the supply to the controller.

- Press “Reset” button on controller to disengage brakes of the motor.

- Now, you can start operating the teach pendant.

# Robot Homing Procedure

Steps required for robot homing are as follow:

- When switching the controller on for the first time, rotate the knob on the controller clockwise.

- Remove the Emergency Stop if any, by rotating the button clockwise or anti-clockwise.

- Press the Control On button.

- Press the Reset button, which resets the controller.

- The Teach Pendant is also turned on along with robot controller. Log in to ADMIN mode. (PWD:tal123)

- Put the teach pendant to Manual mode. (If the teach pendant is not responding then put it to idle/auto mode and then again to manual mode.)

- Go to jog window and select Joint mode.

- Take the robot to its pre-homing position as shown, by jogging the respective joint axes.

- Click on the Home button. (Home tab will be disabled if homing is already done. Then only you can proceed working on any of the cells.)

- A pop up message will be displayed to ask if the system is ready for the homing process. Click yes.

- Wait until the homing process is completed. After homing has been completed, robot will face the table.

- The system is ready to use voice control application through GUI window or to do programming by using teach pendant.

# Operating Teach Pendant

- Teach pendant will get on along with robot controller and other components.

- The robot programming window will open on the teach pendant. Otherwise on the desktop, click on teachpendant.exe and enter the password- “tal123” for admin profile.

- Click on new program or open program as per the need.

- Go to ‘jog’ window on the teach pendant to jog the robot.

- For jogging of the robot, move the black knob on teach pendant to ‘manual’ position (in anti-clockwise direction).

- Note: Homing must be performed before performing any programming task on the robot.

Manual/ Idle/ Auto modes

Linear mode selection for jogging      Joint mode selection for jogging

- In the ‘program’ window, one can use various robot commands available on teach pendant to program the robot for any pick and place operation.

- After programming is done, go to installation tab and select save project. Save the project with some name.

- After saving, go to program tab again and select “robot program” and then on “command”.

- Turn the black knob on teach pendant to “auto mode” (clockwise direction). “Run” option will be displayed in that window.

- The robot program will go on endlessly unless you click on “stop” displayed in the teach pendant or operator box.

- You can even pause and resume the program using the relevant icons on teach pendant and operator box.